CEILING TRAINEE

GUIDE

Pre-Apprentices and Apprentices up to 1 year

This Training Guide belongs to:  ________________________________________

Hire Date: ________________

Trainer(s): ________________________________________________

Job Description:  Carpenter – Acoustical Ceiling

WHAT YOU GET TO DO:

- Perform various carpentry tasks such as installing suspension grid and hanger wires, measuring and cutting ceiling tile to size, installing acoustical and/or custom wall panels, and installing specialty/custom ceiling finishes.

- Read and interpret blueprints to ensure accurate execution of projects.

- Collaborate with team members and work independently to complete projects within specified timelines

- Use tools such as screw guns, lasers and various other hand tools.

- Adhere to safety protocols and maintain a clean and organized work area.

- Comply with company policies and perform other duties as assigned.

WHAT WE WANT FROM YOU:

- Ability to read & understand blueprints and measurements

- Comfort using hand tools

- Ability to operate different types of aerial equipment after required training is provided

- Ability to walk, perform work above head, and climb stairs & ladders as required by the job site

- Strong problem-solving skills and attention to detail.

WHAT WE VALUE:

- Preferably 1 year of construction related experience

- Passion for Safety & Quality of work

- Positive attitude that promotes ALC Culture

- Good communication skills and the ability to work both as a team and independently

On-the-Job Training (OJT): The Trainee is given this list of tasks to be learned and completed in accordance to performance standards.  A Trainer has the same list to guide the trainee in task performance until both feel the learner is ready to do the tasks individually and up to safety and quality standards.  When all tasks have been signed off by the trainer and trainee, the training is completed.

Intent: To accelerate the proficiency of ceiling installers, to provide coaching on tasks deemed critical to the trade, to provide direction and guidance and to provide a means to evaluate ceiling installers for the purpose of identifying potential future leaders.

| Task | Page # | Office or Jobsite | OJT Hours | Online Hours | Total Hours |
| --- | --- | --- | --- | --- | --- |
| New Hire Orientation – Day 1 |  |  |  |  |  |
| New Hire Safety Online Training | 4 | Online |  | 0.5 | 0.5 |
| Scaffold Inspection Online Training | 4 | Online |  | 0.5 | 0.5 |
| Arial Lift Online Training | 4 | Online |  | 0.5 | 0.5 |
| Fall Protection Online Training | 5 | Online |  | 0.5 | 0.5 |
| Proper Use of PPE & Set-Up Perry | 5 | Office | 0.5 |  | 0.5 |
| ALC Policies, Health & Safety Manual, Payroll, Union Info | 5 | Office | 1 |  | 1 |
| On-the-Job Training – first week |  |  |  |  |  |
| Site Safety Tour | 6 | Jobsite | 1 |  | 1 |
| Knowledge of Tape Measure | 6 | Jobsite | 0.5 |  | 0.5 |
| Proper use of Powder Actuated Tools | 7 | Jobsite | 0.5 |  | 0.5 |
| Proper Use of Scaffold/Mobile Scaffold/Ladder | 7 | Jobsite | 1 |  | 1 |
| Proper Use of Elevated Work Platforms | 9 | Jobsite | 1.5 |  | 1.5 |
| Knowledge of Ceiling Materials | 10 | Jobsite | 1 |  | 1 |
| Proper Use of Stilts | 11 | Jobsite | 1 |  | 1 |
| Installing Ceiling Wires | 12 | Jobsite | 5 |  | 5 |
| Installing Ceiling Grid | 13 | Jobsite | 10 |  | 10 |
| Installing Ceiling Tiles | 14 | Jobsite | 2 |  | 2 |
| Caulking/Painting & Touch-Ups | 15 | Jobsite | 1 |  | 1 |
| Evaluation – after first week |  |  |  |  |  |
| Trainee Evaluation by Trainer | 17 | Jobsite | 0.5 |  | 0.5 |
| Trainer Evaluation by Trainee | 19 | Jobsite | 0.5 |  | 0.5 |
| Self-Paced Online Training – within first month |  |  |  |  |  |
| Anti-Harassment Online Training | 21 | Online |  | 0.5 | 0.5 |
| Heat Illness Online Training | 21 | Online |  | 0.5 | 0.5 |
| TOTAL |  |  | 27 | 3 | 30 |

NEW HIRE ORIENTATION   -  3.5 HOURS TOTAL

TASK: NEW HIRE SAFETY  - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online New Hire Safety Orientation on or before their first day before conducting any on-the-job activities.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: SCAFFOLD INSPECTION - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Scaffold Inspection training on or before their first day before using a baker or scaffold on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: ARIAL LIFTS TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Arial Lift training on or before their first day before being on an Arial lift on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: FALL PROTECTION - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Fall Protection training on or before their first day before being on a boom lift or any other elevated platform that requires fall protection equipment on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: PROPER USE OF PPE & PERRY SCAFFOLD SET-UP

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will receive a demonstration of how to put on their Fall Protection Harnesses. Trainer will ensure employee is able to properly use the harness and that they have the proper fit. Trainer will demonstrate and have employee practice setting-up a baker scaffold including setting up the legs and ensuring board is level.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: ALC ORIENTATION (POLICIES, PPE, UNION, PAYROLL)

TIME: 1 HOUR

All new employees will receive a demonstration of how to put on their Fall Protection Harnesses. Trainer will ensure employee is able to properly use the harness and that they have the proper fit. Trainer will demonstrate and have employee practice setting-up a baker scaffold including setting up the legs and ensuring board is level.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: SITE SAFETY TOUR

TIME: 1 HOUR

ON-THE-JOB TRAINING – 25 HOURS

Trainer Objective: Walk the trainee around the jobsite to explain important safety topics and potential hazards as they apply to the work being performed on the job.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: KNOWLEDGE OF A TAPE MEASURE

TIME: 30 MINUTES

Trainer Objective:  The trainee will demonstrate ability to read a tape measure, know the different types for different jobs, & measure to the nearest 1/16”.

Evaluation Items:

- Identify the following marks on a tape:

  - 4’ – 1/8”

  - 1’ – 1/16”

  - 7’ – 1/4”

  - 2’ – 7/16”

  - 12’3 – 7/8”

  - 5’ – 3/16”

- Identify the proper tape for:

  - Framing – 25’ X 1” ;  Layout – 100’

- Use tape measure correctly when checking square with 3-4-5, 6-8-10, 12-16-20 method.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF POWDER ACTUATED TOOLS

TIME: 30 MINUTES

Trainer Objective: Trainee should be able to demonstrate the following steps for each model safely.

Evaluation Items:

- Hilti DX 351 & Hilti DX 460

  - Operator and bystander must wear eye protection, hard hats, and as necessary ear protection

  - Insert fastener into baseplate – ensure it clicks into place

  - Insert cartridge strip all the way into the cartridge strip guide way

    - DX 460 - adjust power level

  - Press the tool onto the surface at a right angle

  - Squeeze the trigger smoothly

  - Wait 30 seconds before removing a misfired cartridge- keep the tool pointed in a safe direction.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF SCAFFOLD/MOBILE SCAFFOLDS & LADDERS

TIME: 1 HOUR

Trainer Objective: Verify the trainee’s knowledge of safe scaffold and ladder practices and usage for different conditions.

Evaluation Items:

- Demonstrate scaffold safe work practices

  - Inspect scaffold equipment for any wear that would compromise it

  - Use J-line in open area

  - Locks and gravity pins in place

  - Fully planks walking area

  - Use guardrails at 4’ on Web-Tex

  - Use guardrails at 6’ in J-Line

  - No self-propelling

  - Scaffold must be moved by a ground person

  - Ensure clean and level floor with no holes or drop offs

  - Position scaffold at the correct height to avoid awkward positions or overreaching

  - Use personal fall protection when guardrails are not in place, maximum height on Web-Tex 8’ and J-line 16’.

  - Tagging System:

    - Be informed and aware that red tag means “Do Not Use”

    - Be informed and aware that green tag needs to be inspected and signed by competent person before use.

- Demonstrate baker (perry) scaffold safe work practices:

  - Inspect the baker scaffold for any wear that would comprise it

  - Ensure the wheels are locked

  - Inspect the board to make sure it’s level, not warped, and secured.

- Demonstrate proper ladder safety procedures:

  - Inspect the ladder for any wear and tear that would comprise it

  - Use 3-point climb

  - Place ladder on level surface

  - Face ladder, keeping body within the boundaries of ladder

  - Ladder at maximum angle to work

  - For extending ladders – trainee needs to secure top and bottom with top extending 36” past the landing

  - Explain procedure for removing defective ladders from service.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF ELEVATED WORK PLATFORMS

TIME: 1.5 HOURS

Trainer Objective: Verify the trainee’s knowledge of safe practices and usage of elevated work platforms in various workplace conditions.

Evaluation Items:

- Inspection of ariel lift

  - Check mechanical condition

  - Battery and hydraulic fluids

  - Tire condition

  - Controls

  - Emergency Release

  - Reports all problems to Foreman and does not use lift.

- Operation of arial lift

  - Boom Lifts: Use required fall protection

  - Scissor Lifts:

  - Understand and follow all warning labels on lift

  - Safety chain must be latched while on aerial lift

  - Proper use of fall protection

  - Does not drive elevated aerial lift near any hazard

  - Operate aerial lift at proper load capacity

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: KNOWLEDGE OF CEILING MATERIALS

TIME: 1 HOUR

Trainer Objective: The trainee will demonstrate knowledge of ceiling materials and demonstrate knowledge of handling material in a safe manner.

Evaluation Items:

- Identify the 3 common types of ceiling grid (USG, Armstrong, Rockfon)

- Identify common sizes and types of grid:

- Identify cross tees and main tees, wall mold (15/16th, 9/16th, shadow mold)

- Demonstrate knowledge of ceiling tile vocabulary:

Safety Items:

- Wear required PPE: hard hat, safety glasses and gloves

- Demonstrate proper lifting techniques:

  - Test load

  - Center body to load

  - Raise chin prior to lifting

  - Lift with legs keeping natural curve in back

  - Use entire body and move feed in direction of load

  - If needed, get help with the load.

- Demonstrate safe handling of metals, using gloves, always lifting, not sliding, never handle sharp edges.

- Use proper material handling procedures:

  - Load flat dolly with load centered and load below eye level

  - Use care when handling ceiling tiles as they are often fragile

  - Use care when handling grid materials as they are often sharp

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF STILTS

TIME: 1 HOUR

Trainer Objective: The trainee will practice using stilts with support from the trainer to ensure safe practices.

Evaluation Items:

- Inspect stilts before each use looking for deterioration of footpads, straps, buckles, and structural members

- Work area clear of clutter, debris, and obstructions

- Take small steps and keep weight above your feet

- Remove stilts prior to using stairs

- Keep work close and within reach

- Be careful when changing directions

- Do not use phone while walking on stilts

- Raise guardrails to appropriate height when near an elevated edge

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: INSTALLING CEILING WIRES

TIME: 5 HOURS

Trainer Objective: Trainee will demonstrate the ability to safely install anchoring system, set-up equipment needed for the job, discuss splay limitations, and install ceiling wires according to layout.

Standards:

- Wires must be 6” off wall and spaced 4’ on center or per prints

- Must be hanging in a straight line

- Trapezed properly

- Length of wire should be correct per elevation

- Wires should never be hung from mechanicals

Evaluation Items:

- Inspect area for hazards, abate or secure area

- Have all tools and materials at hand

- Discuss splay limitations

- Determine where trapezes are needed

- Demonstrate ability to anchor system: twist, shot, or screw (i.e. concrete, lag eye screws, or bar joist)

- Set up materials and equipment

- Install wire per layout, starting first wire 6” from wall and 4’ on center thereafter

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: INSTALLING CEILING GRID

TIME: 10 HOURS

Trainer Objective:  The trainee will demonstrate the ability to safely set-up equipment, layout a room using RCP (Reflected Ceiling Plan), run a string line, square grid using formula, install grid and level, and use hole punch to accommodate fixtures.

Evaluation Items:

- Inspect area for hazards

- Discuss square grid formula:

  - 2X4 grid = 52-5/16” diagonal for 15/16th grid

  - 2X2 grid = 32-1/2” diagonal for 15/16th grid

  - Add ½ inch for 9/16th grid

- Layout a room per RCP (Reflective Ceiling Plan).

- Run a string line

- Cut grid to string line

- Square grid with equal borders on all four sides

- Set up a laser at correct elevation per RCP plan

- Be able to level ceiling after grid installation putting three twists within three inches from top of main T’s.

- Demonstrate ability to use hole punch for off center fixtures (i.e., lights, grills, etc)

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: INSTALLING CEILING TILES

TIME: 2 HOURS

Trainer Objective: The trainee will be able to identify directional and non-directional tile, take measurements of tile, cut reveals with clean edge, cut tight penetrations, and seat tile properly in grid.

Tools Needed:

- Ceiling tile

- Baker scaffold, lift, or stilts

- Utility knife

- Measuring Tape

- Circle cutter

- Keyhole saw

Standards:

- Borders are neat, with no damaged tile

- Tile is properly seating

- Directional tile is installed accordingly

- Smooth, clean cuts on tegular tile

- Penetrations cut with no gaps

- No broken, nicked, or soiled tiles

Evaluation Items:

- Verify correct tile for location per plans

- Get measurements for tile

- Cut reveal and square edge tile using a sharp knife for clean cuts

- Cut penetrations as needed (i.e. sprinkler heads, can lights, etc.)

- Install tile without damaging it

- Seat tile in grid properly

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: CAULKING/ PAINTING & TOUCH-UPS

TIME: 1 HOUR

Trainer Objective: Trainee will demonstrate knowledge of types of caulking; will perform high work according to safety best practices and will apply caulking to standards.

Trainee should be given the following materials to perform the work:

- Caulking gun and caulk

- Dust mask (when required)

- Touch-up paint

Standards:

- No sags, no gaps, clean and smooth, flush with rock

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: EVALUATION OF TRAINEE

TIME: 30 MINUTES

Trainee: ______________________                                Date: _________________

Trainer(s): ______________________________________

| Performance | Excellent | Good | Fair | Poor | Comments |
| --- | --- | --- | --- | --- | --- |
| Safe Worker |  |  |  |  |  |
| Job Knowledge |  |  |  |  |  |
| Productivity |  |  |  |  |  |
| Quality of Work |  |  |  |  |  |
| Technical Skills |  |  |  |  |  |
| Enthusiasm |  |  |  |  |  |
| Trainability |  |  |  |  |  |
| Attitude |  |  |  |  |  |
| Initiative |  |  |  |  |  |
| Culture Fit |  |  |  |  |  |
| Punctuality |  |  |  |  |  |
| Attendance |  |  |  |  |  |
| Communication |  |  |  |  |  |
| Overall Rating |  |  |  |  |  |

Notes: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TASK: EVALUATION OF TRAINING

TIME: 30 MINUTES

Name: ______________________                                Date: _________________

| Training | Excellent | Good | Fair | Poor | Comments |
| --- | --- | --- | --- | --- | --- |
| Training Content |  |  |  |  |  |
| Trainer Preparedness |  |  |  |  |  |
| Safety Emphasis |  |  |  |  |  |
| Patience of Trainer |  |  |  |  |  |
| Trainer Communication |  |  |  |  |  |
| Overall Trainer Rating |  |  |  |  |  |
| Overall Training Rating |  |  |  |  |  |

Notes: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TASK: ANTI-HARASSMENT TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

SELF PACED ONLINE TRAINING – 1 HOUR

All new employees will complete the online Anti-Harassment training within the first 3 months of their employment.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: HEAT ILLNESS TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Heat Illness training within the first 3 months of their employment.

| Completion Date | Trainee Initials |
| --- | --- |