DRYWALL

TRAINEE

GUIDE

Pre-Apprentices and up to 1 Year Apprentices

This Training Guide belongs to:  _________________________________

Hire Date: ________________

Trainer(s): ____________________________________________

Job Description:  Carpenter – Drywall

WHAT YOU GET TO DO:

- Perform various carpentry tasks such as exterior/interior steel stud framing, exterior sheathing, layout of interior walls, hanging drywall, taping drywall, fire stop protection, and occasionally finish carpentry.

- Read and interpret blueprints to ensure accurate execution of projects.

- Collaborate with team members to complete projects within specified timelines.

- Use tools such as screw guns, routers, circular saws, lasers and various other hand tools.

- Adhere to safety protocols and maintain a clean and organized work area.

- Comply with company policies and perform other duties as assigned.

WHAT WE WANT FROM YOU:

- Ability to read & understand blueprints and measurements

- Comfort using hand tools

- Ability to operate different types of aerial equipment after required training is provided

- Ability to lift up to 50 pounds frequently

- Ability to stoop, walk, kneel and climb stairs & ladders as required by the job site

- Strong problem-solving skills and attention to detail.

WHAT WE VALUE:

- Preferably 1 year of construction related experience

- Passion for Safety & Quality of work

- Positive attitude that promotes ALC Culture

- Good communication skills and the ability to work both as a team and independently

On-the-Job Training (OJT): The Trainee is given this list of tasks to be learned and completed in accordance to performance standards.  A Trainer has the same list to guide the trainee in task performance until both feel the learner is ready to do the tasks individually and up to safety and quality standards.  When all tasks have been signed off by the trainer and trainee, the training is complete.

Intent: To accelerate the proficiency of drywall installers, to provide coaching on tasks deemed critical to the trade, to provide direction and guidance and to provide a means to evaluate drywall installers for the purpose of identifying potential future leaders.

| Task | Page # | Office or Jobsite | On-Site Hours | Online Hours | Total Hours |
| --- | --- | --- | --- | --- | --- |
| New Hire Orientation – Day 1 |  |  |  |  |  |
| New Hire Safety Online Training | 4 | Online |  | 0.5 | 0.5 |
| Scaffold Inspection Online Training | 4 | Online |  | 0.5 | 0.5 |
| Arial Lift Online Training | 4 | Online |  | 0.5 | 0.5 |
| Fall Protection Online Training | 5 | Online |  | 0.5 | 0.5 |
| Proper Use of PPE & Set-Up Perry | 5 | Office | 0.5 |  | 0.5 |
| ALC Policies, Health & Safety Manual, Payroll, Union Info | 5 | Office | 1 |  | 1 |
| On-the-Job Training – first week |  |  |  |  |  |
| Site Safety Tour | 6 | Jobsite | 1.5 |  | 1.5 |
| Knowledge of Tape Measure | 6 | Jobsite | 0.5 |  | 0.5 |
| Proper Use of Scaffold/Mobile Scaffolds & Ladders | 7 | Jobsite | 1 |  | 1 |
| Proper Use of Elevated Work Platforms | 8 | Jobsite | 1.5 |  | 1.5 |
| Knowledge of Drywall Materials | 9 | Jobsite | 1 |  | 1 |
| Drywall Handling/Lifting Techniques | 10 | Jobsite | 1 |  | 1 |
| Installing Drywall | 11 | Jobsite | 16 |  | 16 |
| Caulking | 13 | Jobsite | 2 |  | 2 |
| Evaluations – end of first week |  |  |  |  |  |
| Trainee Evaluation by Trainer | 15 | Jobsite | 0.5 |  | 0.5 |
| Trainer Evaluation by Trainee | 17 | Jobsite | 0.5 |  | 0.5 |
| Self-Paced Online Training - within first month |  |  |  |  |  |
| Anti-Harassment Online Training | 19 | Online |  | 0.5 | 0.5 |
| Heat Illness Online Training | 19 | Online |  | 0.5 | 0.5 |
| TOTAL |  |  | 27 | 3 | 30 |

NEW HIRE ORIENTATION   -  3.5 HOURS TOTAL

TASK: NEW HIRE SAFETY  - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online New Hire Safety Orientation on or before their first day before conducting any on-the-job activities.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: SCAFFOLD INSPECTION - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Scaffold Inspection training on or before their first day before using a baker or scaffold on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: ARIAL LIFTS TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Arial Lift training on or before their first day before being on an Arial lift on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: FALL PROTECTION - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Fall Protection training on or before their first day before being on a boom lift or any other elevated platform that requires fall protection equipment on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: PROPER USE OF PPE & PERRY SCAFFOLD SET-UP

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will receive a demonstration of how to put on their Fall Protection Harnesses. Trainer will ensure employee is able to properly use the harness and that they have the proper fit. Trainer will demonstrate and have employee practice setting-up a baker scaffold including setting up the legs, weeks, and securing board.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: ALC ORIENTATION (POLICIES, PPE, UNION, PAYROLL)

TIME: 1 HOUR

All new employees will receive a demonstration of how to put on their Fall Protection Harnesses. Trainer will ensure employee is able to properly use the harness and that they have the proper fit. Trainer will demonstrate and have employee practice setting-up a baker scaffold including setting up the legs and ensuring board is level.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: SITE SAFETY TOUR

TIME: 1 HOUR & 30 MINUTES

ON-THE-JOB TRAINING – 25 HOURS

Trainer Objective:  Walk the trainee around the jobsite to explain important safety topics & potential hazards as they apply to the work being performed on the job.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: KNOWLEDGE OF A TAPE MEASURE

TIME: 30 MINUTES

Trainer Objective:  The trainee will demonstrate ability to read a tape measure, different types of tape measures for different jobs, measure to the nearest 1/16”.

Evaluation Items:

- Identify the following marks on a tape:

  - 4’ – 1/8”

  - 1’ – 1/16”

  - 7’ – 1/4”

  - 2’ – 7/16”

  - 12’3 – 7/8”

  - 5’ – 3/16”

- Identify the proper tape for:

  - Framing – 25’ X 1” ;  Layout – 100’

- Use tape measure correctly when checking square: 3-4-5, 6-8-10, 12-16-20 method.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF SCAFFOLD/MOBILE SCAFFOLDS & LADDERS

TIME: 1 HOUR

Trainer Objective: Verify the trainee’s knowledge of safe scaffold and ladder practices and usage for different conditions.

Evaluation Items:

- Demonstrate scaffold safe work practices

  - Inspect scaffold equipment for any wear that would compromise it

  - Locks and gravity pins in place

  - Fully planks walking area

  - Use guardrails at 4’ on Web-Tex and at 6’ in J-Line

  - Scaffold must be moved by a ground person

  - Ensure clean and level floor with no holes or drop offs

  - Position scaffold at the correct height to avoid awkward positions or overreaching

  - Use personal fall protection when guardrails are not in place, maximum height on Web-Tex 8’ and J-line 16’.

  - Tagging System:

    - Be informed and aware that red tag means “Do Not Use”

    - Be informed and aware that green tag needs to be inspected and signed by competent person before use.

- Demonstrate baker (perry) scaffold safe work practices:

  - Inspect the baker scaffold for any wear that would comprise it

  - Ensure the wheels are locked

  - Inspect the board to make sure it’s level, not warped, and secured.

- Demonstrate proper ladder safety procedures:

  - Inspect the ladder for any wear and tear that would comprise it

  - Place ladder on level surface & use 3-point climb

  - Face ladder, keeping body within the boundaries of ladder

  - For extending ladders – trainee needs to secure top and bottom with top extending 36” past the landing

  - Explain procedure for removing defective ladders from service.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF ELEVATED WORK PLATFORMS

TIME: 1.5 HOURS

Trainer Objective: Verify the trainee’s knowledge of safe practices and usage of elevated work platforms in various workplace conditions.

Evaluation Items:

- Inspection of ariel lift

  - Check mechanical condition

  - Battery and hydraulic fluids

  - Tire condition

  - Controls

  - Emergency Release

  - Reports all problems to Foreman and does not use lift.

- Operation of arial lift

  - Boom Lifts: Use required fall protection

  - Understand and follow all warning labels on lift

  - Safety chain must be latched while on aerial lift

  - Proper use of fall protection

  - Does not drive elevated aerial lift near any hazard

  - Operate aerial lift at proper load capacity

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: KNOWLEDGE OF DRYWALL MATERIALS

TIME: 1 HOUR

Trainer Objective: The trainee will demonstrate knowledge of materials, be able to identify material and its common name(s) to include drywall and metal materials.

Evaluation Items:

- Identify the following drywall types:

  - Dens Shield

  - DensArmor Plux

  - WR – Water Resistant Board

  - Abuse Board

  - Type X

  - Type C

- Recognize and recite the different names for same material:

  - CRC – black iron

  - Shaft Liner – core board

  - Angle metal – 2x2, shiny 90

  - Flat metal – Metal backing, flat strap

  - Cement Board – Durarock or backer board

  - Sheetrock, drywall, or wall board

  - Furring channel – hat channel, DWC “drywall channel”

  - RCI – Sound caulk, Hilti red caulk or fire caulk

  - Drill points – Teks

  - Shaft Screws – laminators

- Identify 1 – 5/8”,  3 – 5/8”, 4” and 6” and identify gauge metals

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

Trainer Objective: The trainee will demonstrate

TASK: DRYWALL HANDLING/LIFTING TECHNIQUES

TIME: 1 HOUR

knowledge of safety, equipment evaluation and use, teamwork, and drywall dolly moving and handling techniques.

Evaluation Items:

- Wear required PPE; hard hat, safety glasses

- Demonstrate the following proper lifting techniques

  - Test load

  - Center body to load

  - Raise chin prior to lifting

  - Lift with legs to keep natural curve in back

  - Use entire body and move feet in direction of load

  - If needed, get help with the load

- Demonstrate safe handling of metals using gloves, always lifting not sliding, never handle sharp edges

- Survey path of dolly for obstacles or hazards

- Load dolly with face side out, weight of load on non-turning wheel

Inspection:

- Inspect dolly checking to ensure wheels are stable and in good condition

- Explain procedure for removing dolly from service

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: INSTALLING DRYWALL

TIME: 16 HOURS

Trainer Objective: Trainee will demonstrate proper cutting, marking, trimming, and installation techniques for drywall to include correct positioning of materials for work.

Trainee should be given the following materials to perform the work:

Standards:

- Walls are plumb, level, and square

- Screws are set, but not breaking through paper

- Wallboard attached to bottom track

- Joints are tight and staggered

- Screw spacing 8” on joint; 16” on center (or per specs)

- Drywall does not touch the floor or deck.  Check specifications for required distance.  This accommodates deflection and potential damage to sheetrock.

- Factory butt joints are together

- ¾” gap at control joint

- Drywall cut tight around penetrations

- Clean cut edge

- Spacers are removed at bottom of board

- Hollow metal door jambs are plumb

- Aluminum door jamb condition; check door opening width

Evaluation Items:

Measure and Cut:

- Give, receive and understand directions from partner for measurements, cutouts, etc.

- Cut drywall by using a sheetrock knife, a tape measure, and a carpenter’s pencil

- Mark board for proper stud layout: 16” or 24”

- Measure then cut penetrations with keyhole saw or roto zip-router

- Trim off excess burrs on edge of drywall using a hook bill or rasp

Installation:

- Install drywall using proper screw spacing and joint stagger

- Attach drywall with screws.  Screws are set, but not breaking through paper

- Install drywall in direction of stud layout

- Cut and install drywall around door frames

- Cut drywall tight around penetrations

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: CAULKING

TIME: 2 HOURS

Trainer Objective: Trainee will demonstrate knowledge of types of caulking; will perform high work according to safety best practices and will apply caulking to standards.

Trainee should be given the following materials to perform the work:

- Caulking gun and caulk

- Putty knife or spray gun (when required)

- Dust mask

Standards:

- No sags, no gaps, clean and smooth, flush with rock

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: EVALUATION OF TRAINEE

TIME: 30 MINUTES

Trainee: ______________________                                Date: _________________

Trainer(s): ______________________________________

| Performance | Excellent | Good | Fair | Poor | Comments |
| --- | --- | --- | --- | --- | --- |
| Safe Worker |  |  |  |  |  |
| Job Knowledge |  |  |  |  |  |
| Productivity |  |  |  |  |  |
| Quality of Work |  |  |  |  |  |
| Technical Skills |  |  |  |  |  |
| Enthusiasm |  |  |  |  |  |
| Trainability |  |  |  |  |  |
| Attitude |  |  |  |  |  |
| Initiative |  |  |  |  |  |
| Culture Fit |  |  |  |  |  |
| Punctuality |  |  |  |  |  |
| Attendance |  |  |  |  |  |
| Communication |  |  |  |  |  |
| Overall Rating |  |  |  |  |  |

Notes: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TASK: EVALUATION OF TRAINING

TIME: 30 MINUTES

Name: ______________________                                Date: _________________

| Training | Excellent | Good | Fair | Poor | Comments |
| --- | --- | --- | --- | --- | --- |
| Training Content |  |  |  |  |  |
| Trainer Preparedness |  |  |  |  |  |
| Safety Emphasis |  |  |  |  |  |
| Patience of Trainer |  |  |  |  |  |
| Trainer Communication |  |  |  |  |  |
| Overall Trainer Rating |  |  |  |  |  |
| Overall Training Rating |  |  |  |  |  |

Notes: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TASK: ANTI-HARASSMENT TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

SELF PACED ONLINE TRAINING – 1 HOUR

All new employees will complete the online Anti-Harassment training within the first 3 months of their employment.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: HEAT ILLNESS TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Heat Illness training within the first 3 months of their employment.

| Completion Date | Trainee Initials |
| --- | --- |