EXTERIOR INSULATION & FINISHING SYSTMES

(EIFS)

TRAINEE GUIDE

Pre-Apprentices and Apprentices up to 1 Year

This Training Guide belongs to:  ______________________________

Hire Date: ________________

Trainer(s): _____________________________________

Job Description: Bricklayers & Carpenters – EIFS Installer

WHAT YOU GET TO DO:

- Install and complete all steps of EIFS installation including, foam installation, rasping, installing grooves or bands, basecoat and mesh application, finish coat application and the final step of caulking.

- Read and interpret blueprints to ensure accurate execution of projects.

- Collaborate with team members to complete projects within specified timelines.

- Use tools such as screw guns, hawk & trowel, and various other hand tools.

- Adhere to safety protocols and maintain a clean and organized work area.

- Comply with company policies and perform other duties as assigned.

WHAT WE WANT FROM YOU:

- Ability to read & understand blueprints and measurements

- Experience using hawk & trowel

- Ability to operate different types of aerial equipment after required training is provided

- Ability to lift up to 80 pounds frequently

- Ability to stoop, walk, kneel and climb stairs & ladders as required by the job site

- Strong problem-solving skills and attention to detail.

WHAT WE VALUE:

- Preferably 1 year of construction related experience

- Passion for Safety & Quality of work

- Positive attitude that promotes ALC Culture

- Good communication skills and the ability to work both as a team and independently

On-the-Job Training (OJT): The Trainee is given this list of tasks to be learned and completed in accordance to performance standards.  A Trainer has the same list to guide the trainee in task performance until both feel the learner is ready to do the tasks individually and up to safety and quality standards.  When all tasks have been signed off by the trainer and trainee, the training is completed.

Intent: To accelerate the proficiency of EIFS installers, to provide coaching on tasks deemed critical to the trade, to provide direction and guidance and to provide a means to evaluate EIFS installers for the purpose of identifying potential future leaders.

| Task | Page # | Office or Jobsite | OJT Hours | Online Hours | Total Hours |
| --- | --- | --- | --- | --- | --- |
| New Hire Orientation – Day 1 |  |  |  |  |  |
| New Hire Safety Online Training | 4 | Online |  | 0.5 | 0.5 |
| Scaffold Inspection Online Training | 4 | Online |  | 0.5 | 0.5 |
| Arial Lift Online Training | 4 | Online |  | 0.5 | 0.5 |
| Fall Protection Online Training | 5 | Online |  | 0.5 | 0.5 |
| Proper Use of PPE & Set-Up Perry | 5 | Office | 0.5 |  | 0.5 |
| ALC Policies, Health & Safety Manual, Payroll, Union Info | 5 | Office | 1 |  | 1 |
| On-the-Job Training – first week |  |  |  |  |  |
| Site Safety Tour | 6 | Jobsite | 1 |  | 1 |
| Knowledge of Tape Measure | 6 | Jobsite | 0.5 |  | 0.5 |
| Proper Use of Scaffold/Mobile Scaffold/Ladder | 7 | Jobsite | 1 |  | 1 |
| Proper Use of Elevated Work Platforms | 8 | Jobsite | 1.5 |  | 1.5 |
| Installing Air Barrier & Metal Flashing | 9 | Jobsite | 2.5 |  | 2.5 |
| Installing EIFS Foam | 10 | Jobsite | 6 |  | 6 |
| Applying Mesh Coat & Base Coat | 11 | Jobsite | 5 |  | 5 |
| Applying Finish coat | 11 | Jobsite | 5 |  | 5 |
| Installing Caulk | 13 | Jobsite | 2 |  | 2 |
| Evaluation – after first week |  |  |  |  |  |
| Trainee Evaluation by Trainer | 15 | Jobsite | 0.5 |  | 0.5 |
| Trainer Evaluation by Trainee | 17 | Jobsite | 0.5 |  | 0.5 |
| Self-Paced Online Training – within first month |  |  |  |  |  |
| Anti-Harassment Online Training | 19 | Online |  | 0.5 | 0.5 |
| Heat Illness Online Training | 19 | Online |  | 0.5 | 0.5 |
| TOTAL |  |  | 27 | 3 | 30 |

NEW HIRE ORIENTATION   -  3.5 HOURS TOTAL

TASK: NEW HIRE SAFETY  - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online New Hire Safety Orientation on or before their first day before conducting any on-the-job activities.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: SCAFFOLD INSPECTION - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Scaffold Inspection training on or before their first day before using a baker or scaffold on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: ARIAL LIFTS TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Arial Lift training on or before their first day before being on an Arial lift on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: FALL PROTECTION - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Fall Protection training on or before their first day before being on a boom lift or any other elevated platform that requires fall protection equipment on-the-job.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: PROPER USE OF PPE & PERRY SCAFFOLD SET-UP

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will receive a demonstration of how to put on their Fall Protection Harnesses. Trainer will ensure employee is able to properly use the harness and that they have the proper fit. Trainer will demonstrate and have employee practice setting-up a baker scaffold including setting up the legs, weeks, and securing board.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: SITE SAFETY TOUR

TIME: 1 HOUR

ON-THE-JOB TRAINING – 25 HOURS

Trainer Objective: Walk the trainee around the jobsite to explain important safety topics and potential hazards as they apply to the work being performed on the job.

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: KNOWLEDGE OF A TAPE MEASURE

TIME: 30 MINUTES

Trainer Objective:  The trainee will demonstrate ability to read a tape measure, know the different types for different jobs & measure to the nearest 1/16”.

Evaluation Items:

- Identify the following marks on a tape:

  - 4’ – 1/8”

  - 1’ – 1/16”

  - 7’ – 1/4”

  - 2’ – 7/16”

  - 12’3 – 7/8”

  - 5’ – 3/16”

- Use tape measure correctly when checking square with 3-4-5, 6-8-10, 12-16-20 method.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF SCAFFOLD/MOBILE SCAFFOLDS & LADDERS

TIME: 1 HOUR

Trainer Objective: Verify the trainee’s knowledge of safe scaffold and ladder practices and usage for different conditions.

Evaluation Items:

- Demonstrate scaffold safe work practices

  - Inspect scaffold equipment for any wear that would compromise it

  - Use J-line in open area

  - Locks and gravity pins in place

  - Fully planks walking area

  - Use guardrails at 4’ on Web-Tex

  - Use guardrails at 6’ in J-Line

  - No self-propelling

  - Scaffold must be moved by a ground person

  - Ensure clean and level floor with no holes or drop offs

  - Position scaffold at the correct height to avoid awkward positions or overreaching

  - Use personal fall protection when guardrails are not in place, maximum height on Web-Tex 8’ and J-line 16’.

  - Tagging System:

    - Be informed and aware that red tag means “Do Not Use”

    - Be informed and aware that green tag needs to be inspected and signed by competent person before use.

- Demonstrate baker (perry) scaffold safe work practices:

  - Inspect the baker scaffold for any wear that would comprise it

  - Ensure the wheels are locked

  - Inspect the board to make sure it’s level, not warped, and secured.

- Demonstrate proper ladder safety procedures:

  - Inspect the ladder for any wear and tear that would comprise it

  - Use 3 point climb

  - Place ladder on level surface

  - Face ladder, keeping body within the boundaries of ladder

  - Ladder at maximum angle to work

  - For extending ladders – trainee needs to secure top and bottom with top extending 36” past the landing

  - Explain procedure for removing defective ladders from service.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: PROPER USE OF ELEVATED WORK PLATFORMS

TIME: 1.5 HOURS

Trainer Objective: Verify the trainee’s knowledge of safe practices and usage of elevated work platforms in various workplace conditions.

Evaluation Items:

- Inspection of ariel lift

  - Check mechanical condition

  - Battery and hydraulic fluids

  - Tire condition

  - Controls

  - Emergency Release

  - Reports all problems to Foreman and does not use lift.

- Operation of arial lift

  - Boom Lifts: Use required fall protection

  - Scissor Lifts: Be able to operate it safely

  - Understand and follow all warning labels on lift

  - Safety chain must be latched while on aerial lift

  - Proper use of fall protection

  - Does not drive elevated aerial lift near any hazard

  - Operate aerial lift at proper load capacity

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: INSTALLING AIR BARRIER & METAL FLASHINGS

TIME: 2 HOURS & 30 MINUTES

Trainer Objective: Verify the trainee’s ability properly inspect and apply air/moisture barrier and proper metal flashings.

Evaluation Items:

- Inspect the substrate to make sure it is acceptable.

  - Showing no signs of deterioration

  - Free of any crumbling or looseness of surface

  - No gaps or voids other than what is necessary for proper installation.

  - No projects or planar irregularities greater than ¼ in in a 4ft radius

  - Dry and clean from oil, dust, dirt, paints, wax, water, frost, etc.

- Have proper scaffoldings, or lifts in place prior to the installation.

- Install the liquid applied air/moisture barrier to the substrate.

  - Wrapping window openings per manufacturer product recommendations

  - Apply air barrier over entire substrate.

- Inspect the air barrier application prior to starting EIFS to make sure it is acceptable.

- Install proper metal flashings at window heads and at base of EIFS locations.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: INSTALLING EIFS FOAM BOARD

TIME: 6 HOURS

Trainer Objective: Verify the trainee can install EPS insulation boards.  Teach the proper cutting techniques and different types of fastening techniques.

Evaluation Items:

- Proper cutting techniques

  - Cut with a knife using a square to guide the cut

  - Table saw

- Types of fastening techniques

  - Adhesively

  - Mechanically

- Applied by either spreading cement basecoat with a notched trowel on the back of the insulation sheets or using mechanical fasteners that screw into wood sheathing.

- Evenly level off the EPS insulation using a handheld rasp.

- Layout the wall with project bands or grooves that get cut into the EPS.

  - Can be found on the plan and applied to the EPS foam.

  - Measurements are crucial in keeping bands and grooves straight and level.

  - Insulation board joints should never align with opening corners.  Leave a uniform ½ inch space between the edge of the EPS and the window, door, etc.

  - At all outside and inside corners always interlock or stagger the insulation board.  Level the EPS board by rasping.  Maintain a minimum thickness of ¾ inch of EPS.  Remove all loose EPS particles from wall surface.

  - All voids must be filled with an insulating material, either spray foam or slivers of EPS board.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: APPLYING BASE COAT & MESH

TIME: 5 HOURS

Trainer Objective: Ensure trainee can apply the base and mesh coat properly, up to ALC quality standards.

Evaluation Items:

- Using a trowel, apply an even layer of base coat approximately 1/16 to 3/32 in thick (check prints) onto the surface of the insulation board.

- Immediately, embed the reinforcing mesh into the fresh base coat, trowelling it from the center and outward to its edges, cutting off any accumulated excess base coat.

- If the mesh pattern is still visible, apply a second skim coat.

- The base coat application should be smooth, approximately 1/16 in thick an free of trowel lines, and no visible signs of the reinforcing mesh.

- Allow a minimum drying time of 24 hours.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: APPLYING FINISH COAT

TIME: 5 HOURS

Trainer Objective: Ensure the trainee can apply the finish coat properly, up to ALC quality standards.

Evaluation Items:

Fundamentals:

- Make sure all samples are approved and signed by the customer before the finish is made.

- Box finish from different batches.

- Run one batch of finish from corner to corner

- Avoid application in direct sunlight

- Always work to a wet edge

- Only stop work at a termination point.

Application:

- Apply finish coat with a clean stainless-steel trowel.

- The thickness of the finish application should equal the size of the product’s aggregate.

- To ensure consistency in the application, all applicators should use the same motion.

- During texturing, continuously remove any buildup of finish on the plastic float.

  - Do not wet or clean the float with water while texturing.  The additional water from the float may result in a visible color difference when the finish application dries.

- After the finish coat has cured, backer rod and sealants are to be applied at the EIFS system terminations.  Such as the windows, control joints and at the base of the EIFS where it meets the bottom flashing.

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: CAULKING

TIME: 2 HOURS

Trainer Objective: Trainee will demonstrate knowledge of types of caulking; will perform high work according to safety best practices and will apply caulking to standards.

Trainee should be given the following materials to perform the work:

- Caulking gun and caulk

- Putty knife or spray gun (when required)

- Dust mask

Standards:

- No sags, no gaps, clean and smooth, flush with rock

When trainee has all evaluation items checked off, initial and date the task completion below:

| Completion Date | Trainee Initials | Trainer Initials |
| --- | --- | --- |

TASK: EVALUATION OF TRAINEE

TIME: 30 MINUTES

Trainee: ______________________                                Date: _________________

Trainer(s): ______________________________________

| Performance | Excellent | Good | Fair | Poor | Comments |
| --- | --- | --- | --- | --- | --- |
| Safe Worker |  |  |  |  |  |
| Job Knowledge |  |  |  |  |  |
| Productivity |  |  |  |  |  |
| Quality of Work |  |  |  |  |  |
| Technical Skills |  |  |  |  |  |
| Enthusiasm |  |  |  |  |  |
| Trainability |  |  |  |  |  |
| Attitude |  |  |  |  |  |
| Initiative |  |  |  |  |  |
| Culture Fit |  |  |  |  |  |
| Punctuality |  |  |  |  |  |
| Attendance |  |  |  |  |  |
| Communication |  |  |  |  |  |
| Overall Rating |  |  |  |  |  |

Notes: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TASK: EVALUATION OF TRAINING

TIME: 30 MINUTES

Name: ______________________                                Date: _________________

| Training | Excellent | Good | Fair | Poor | Comments |
| --- | --- | --- | --- | --- | --- |
| Training Content |  |  |  |  |  |
| Trainer Preparedness |  |  |  |  |  |
| Safety Emphasis |  |  |  |  |  |
| Patience of Trainer |  |  |  |  |  |
| Trainer Communication |  |  |  |  |  |
| Overall Trainer Rating |  |  |  |  |  |
| Overall Training Rating |  |  |  |  |  |

Notes: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TASK: ANTI-HARASSMENT TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

SELF PACED ONLINE TRAINING – 1 HOUR

All new employees will complete the online Anti-Harassment training within the first 3 months of their employment.

| Completion Date | Trainee Initials |
| --- | --- |

TASK: HEAT ILLNESS TRAINING - ONLINE

TIME: 30 MINUTES

ONLINE: KPA PORTAL

All new employees will complete the online Heat Illness training within the first 3 months of their employment.

| Completion Date | Trainee Initials |
| --- | --- |